Tensegrity Simulator — Project Plan and Grading Breakdown

Alizée  ·  Aerospace Engineering, University of Houston  ·  Plan submitted 6 October 2026  ·  All deliverables complete by 3 December 2026

# 1. Project summary

This project is an interactive tensegrity structure simulator built from scratch in Babylon.js with WebXR support. It was begun earlier this year and has been developed continuously since: the language and toolchain work, the rendering pipeline, the structure catalogue architecture, the physics solver and the WebXR session are all original work produced for this project, and they are included below as graded subtasks rather than treated as pre-existing material.

The objective is a design tool grounded in minimal-mass tensegrity theory (Skelton & de Oliveira, Tensegrity Systems, Chapters 2–4): every member sized against Euler buckling and yielding, a valid self-stress state, and real-time display of which members approach failure and what a design costs in mass. The distinctive element is this live constraint feedback during interactive manipulation — minimal-mass optimisation is well covered as a batch computation, but not as something a user explores by hand in VR.

The twelve subtasks span the whole arc of the work: 1 to 5 are complete and demonstrable now, 6 to 12 are scheduled across the remaining nine weeks, ordered so each is demonstrable alone and the riskiest is attempted early.

# 2. Subtask summary and point allocation

| # | Subtask | Points | Status / due |
| --- | --- | --- | --- |
| 1 | Language and toolchain foundation | 5 | Complete |
| 2 | Scene, rendering pipeline and lunar environment | 6 | Complete |
| 3 | Structure catalogue architecture and first three structures | 8 | Complete |
| 4 | Physics solver: Verlet integration and constraint projection | 7 | Complete |
| 5 | Member force readout and WebXR session | 4 | Complete |
| 6 | Member sizing engine | 10 | 13 Oct |
| 7 | Equilibrium solver and valid self-stress | 15 | 26 Oct |
| 8 | Minimal-mass model and optimisation panel | 12 | 2 Nov |
| 9 | Structure catalogue expansion | 8 | 9 Nov |
| 10 | Interactive manipulation with live failure feedback | 15 | 14 Nov |
| 11 | Validation suite | 5 | 17 Nov |
| 12 | Report, demonstration video and presentation | 5 | 3 Dec |
|  | TOTAL | 100 |  |

Thirty points sit with the delivered foundation — several months of work, and what everything else is built on. The remaining seventy are weighted by difficulty and dependency: subtask 7 is the keystone, since a general equilibrium solver gives every catalogue structure correct member forces, and subtask 10 is the contribution the project is built around.

# 3. Delivered subtasks (1–5)

1. Language and toolchain foundation (5 pts) — TypeScript learned and applied from scratch, with the Node toolchain, VS Code configuration and a live development server. Evidence: the project compiles under strict TypeScript.

2. Scene, rendering pipeline and lunar environment (6 pts) — Babylon.js scene with camera control, lighting, a lunar surface environment, and procedural mesh generation building strut cylinders and cable lines from an arbitrary node list. Evidence: any catalogue structure renders and is inspectable from any angle.

3. Structure catalogue architecture and first three structures (8 pts) — A data-driven structure format holding parameters, a node-generation function and separate strut and cable connectivity lists, so new structures are added as data rather than code. Three implemented against it: 3-bar prism, T-Bar, D-Bar, with correct geometry including the prism twist. Evidence: structures switch at runtime with full mesh disposal and rebuild.

4. Physics solver: Verlet integration and constraint projection (7 pts) — A position-based dynamics solver with Verlet integration, distance constraints projected over configurable sub-steps, damping, gravity and pinned nodes. Evidence: structures settle stably and respond to node displacement without drift.

5. Member force readout and WebXR session (4 pts) — Per-member force computed from constraint extension and displayed as scaled arrows, plus a WebXR session with controller input tested on headset hardware. Evidence: forces update live and the scene is enterable in VR.

# 4. Remaining subtasks (6–12): deliverables and acceptance criteria

Subtask 6 — Member sizing engine (10 points, due 13 October)

Deliverable: A module computing the radius each member needs: Euler buckling for bars, yielding for cables, whichever governs for a bar. Includes member and total mass, the design parameter ε, and a per-member utilisation ratio. Material catalogue extended with Young's modulus and yield stress.

Done when: Unit tests reproduce the textbook benchmark — a 6 m steel bar carrying 20 kN needs r = 25.9 mm and masses 99.5 kg — and strut mesh diameters are driven by computed radii rather than a fixed visual thickness. Estimated effort 8 to 10 hours.

Subtask 7 — Equilibrium solver and valid self-stress (15 points, due 26 October)

Deliverable: Assembly of the equilibrium matrix A (3 × nodes by members, each column the unit vector along a member) and a least-norm solve for the force-density vector, giving member forces for any structure and load case. Prestress assigned per member group from analytic force densities rather than a single uniform value.

Done when: The equilibrium residual is numerically zero for the 3-bar prism under self-stress alone, loaded T-Bar forces match closed-form values, and the uniform-prestress assumption is removed. Estimated effort 14 to 18 hours.

Risk note: Highest-risk subtask, therefore scheduled first. Fallback: supply analytic force densities for the existing three structures, sufficient for subtasks 8 and 10 though not reusable.

Subtask 8 — Minimal-mass model and optimisation panel (12 points, due 2 November)

Deliverable: Mass-ratio functions for T-Bar and D-Bar self-similar columns and stacked prism columns, the optimal complexity, and the yield-crossover ceiling beyond which the buckling model is invalid. A panel plots mass ratio against complexity with the current design, the optimum and the invalid region marked.

Done when: Sliders update the plot live, computed optima match hand-checked closed-form values for at least three cases, and the panel refuses to recommend complexity above the yield ceiling. Estimated effort 10 to 12 hours.

Subtask 9 — Structure catalogue expansion (8 points, due 9 November)

Deliverable: Prism twist replaced by its analytic form α = π/2 − π/p, making 4-bar and 6-bar prisms correct automatically; a stacked prism column of configurable height; and a planar Michell topology generator for bending loads.

Done when: At least six catalogue entries load, render and return member forces without error, and the twist angle is computed rather than hard-coded. Estimated effort 8 to 10 hours.

Subtask 10 — Interactive manipulation with live failure feedback (15 points, due 14 November)

Deliverable: The user grabs a node and moves it; the solver re-equilibrates each frame; members are coloured by utilisation (green below 60 %, amber to 90 %, red above) and utilisation 1.0 triggers a failure indication. Mass and peak utilisation display continuously.

Done when: A recorded session shows manipulation with colouring and readouts updating at interactive frame rate, and one member driven deliberately to failure. Estimated effort 16 to 20 hours.

Fallback: If headset integration proves unstable the identical interaction ships as desktop mouse-drag; the contribution is the live re-equilibration and feedback loop, not the input device.

Subtask 11 — Validation suite (5 points, due 17 November)

Deliverable and acceptance: Automated checks — equilibrium residual near zero for every structure, structure mass agreeing with the closed-form mass ratio, and member count matching the self-similar formula — all run from a single command, with results and any discrepancies recorded in a table for the report. Estimated effort 6 to 8 hours.

Subtask 12 — Report, demonstration video and presentation (5 points, due 3 December)

Deliverable and acceptance: A report covering the theory, architecture, validation results and an explicit limitations section (small displacements, PBD not being finite element analysis, stiffness not computed); a screen-captured demonstration; and the final slides. Complete when all three exist with figures and the validation table. Estimated effort 12 to 14 hours.

# 5. Schedule

| Period | Focus |
| --- | --- |
| Delivered | Subtasks 1 to 5: toolchain, rendering, catalogue architecture, solver, force display, WebXR |
| 6 – 12 Oct | Subtask 6: sizing functions and unit tests |
| 13 – 26 Oct | Subtask 7: equilibrium matrix and least-norm solve; prestress per member group |
| 27 Oct – 9 Nov | Subtask 8: mass curves and optimisation panel; subtask 9: catalogue entries |
| 10 – 14 Nov | Subtask 10: interaction and failure colouring. FEATURE FREEZE 14 November |
| 17 – 28 Nov | Subtask 11: validation runs recorded; Thanksgiving week: writing only, no new code |
| 1 – 3 Dec | Subtask 12: report finalised, video recorded, submitted |

Remaining effort is 75 to 90 hours, or 8 to 10 per week. Feature work stops 14 November, leaving two weeks for validation and writing.

# 6. Risks and mitigations

Solver instability (subtask 7) is scheduled early so a fallback to analytic force densities stays possible. Headset or frame-rate problems (subtask 10) are covered by a desktop mouse-drag fallback using the same solver and feedback path. Stretch goals carry zero points and are attempted only after subtask 11. Documentation sits in protected weeks so a busy November cannot eliminate it.

# 7. Stretch goals (not graded, zero points)

Listed so the direction is clear; no points depend on them. A spherical Michell structure under torsion, tensegrity plates under crushing load, the self-similar fractal generator at complexity three or above, and a one-cable D-Bar deployment demonstration.